# Circuit Set-up Protocol

We started the first semester with a short introduction to circuits:

- You need a current loop.

- Current is measured in series.

- Voltage is measured in parallel.

- Ohm’s law states that V = RI.

We ended up with a more and more detailed procedure in order to have fewer unsuccessful experimental runs. The procedure was improved based on students’ struggles and reminders to “please, state this explicitly”, which really makes it “their” procedure.

As the computer records the data, the main steps are:

- Did students measure the introductory data (emf and external resistance)?

- Did students connect the wires correctly?

- Did students set-up the computer correctly (1 data point per second; end measurement manually)?

We would go through the classroom and confirm that the groups had performed the above three steps. Our check occurs between steps 3) and 4) of the measurement procedure. Only then were students allowed to

- First: start the computer measurement.

- Second: connect the final wire in the circuit to start the current flow.

Once the circuit is connected, watch for the first minute (or come back for a second round) to see that everything seems to be recording properly.

What we learned during the first semester:

- The laptops need to be plugged in and the “battery settings” need to be set to “never shut off when plugged in”. Otherwise, the computers will shut down in the middle of the measurement, as you have not been “wiggling the mouse”.

- Do not use the computer for calculations while it records data. On some computers that seems to overextend the CPU and the measurement gets corrupted.

- If the measurement ends successfully, but does not let you save, you can copy the data into Excel to save it. If this is the case, or the measurement freezes, make sure to check the task master, if the program keeps running as a broken task that needs to get deleted to start with a clean slate. A lot of students are very unfamiliar with this process.

Below are some nice images of the circuit set-up from our students (6 different groups from Spring 2025). My hand-out to the students has a very unattractive image, as I always hope that they make a nice one for their presentations instead of copying mine.

Vernier

E & F

B

D

C

A

A