# Equipment List

Equipment that students brought:

- Laptop computers (these are the students’ personal laptops). Tablets (like i-pads) do not work for recording the data, but have been used successfully for data analysis

- Vernier Graphical Analysis program. This is a free program that the students downloaded and tested on the first lab day. This program runs on Windows and Mac. Older program versions are available that run successfully on older computers.

Equipment that was available in the lab. This was “shared” between different lab days, i.e., this equipment was not in the boxes with the “personal” material for each group.

- Mulitmeter (to measure resistance of load resistor and emf of battery).

- Vernier voltage probe.

- Vernier current probe.

- Two Vernier GoLinks.

- USB-hub: As we have two USB-wires from the two GoLinks.

- USB to USB-C converter, for students whose laptop did not have a USB connector port.

Equipment that was kept in a box labeled for a specific group

- Lab notebook (bought by us, one per group, at the start of the school year for $0.50 each). This is a group notebook that stays in the lab. Students record the necessary data at the start of the lab and a comment at the end of the lab, if the measurement was successful or what has happened. For the rest of the lab, some work is done on the computers, some work is done on worksheets that were eventually taped into the notebook, and some notes are recorded in the notebook.

- One short alligator-clip wire (recommended: A colored wire that is not red or black, as there are already a lot of red and black wires being used).

- Battery Holder (designed for CR2032 batteries, but works fine for our CR2016 batteries that have the same diameter at half the height).

- Batteries: We chose CR2016 batteries, as they have a relatively small battery charge capacity, so that they can be noticeably drained over the course of a lab. We were also looking for a battery for which we could buy a battery holder.

- Resistors with different resistance values (between 100 Ω and 1000 Ω). Students kept ‘their’ resistor(s), connected to the colored alligator-clip wire. As resistors have a specific value within an allowed variability range, we did not want to introduce an additional variable by using “similar but different” resistors from one week to the next.