# How to Perform the Experiment

## Before building the circuit

- Take out old battery -> put in zip lock bag

- Put in new battery -> lettering visible to you

- Measure emf E of battery (voltage w/out load)

  - E = __________ V (setting: 20 V, DC)

- Measure value of load’s resistance (resistance of blue resistor)

  - Rexternal = __________ Ω (setting: 2 kΩ)

Perform these steps before assembling the circuit.

## Build the circuit

Don’t connect the colored wire to the battery yet. This will be done in step 4.

- Connect the current probe

  - Red wire gets connected to “bump-side” of battery

  - Black wire gets connected to resistor side with no wire on it

- Connect the voltage probe

  - Red wire gets connected to resistor side with black wire on it

  - Black wire gets connected to resistor side with colored wire on it

- Plug current probe and voltage probe into GoLinks. Plug GoLinks into laptop

  - USB ports are available

  - USB-C to USB converter is available

## Set-up the computer

- Screen settings: When plugged in

  - Never turn off screen

  - Never go to sleep

- Start Vernier Graphical Analysis program. 
If circuit with GoLinks has been connected, screen should show two labeled graphs

  - Potential vs time

  - Current vs time

- Zero the two probes

  - On the bottom right, click on the values displayed and select zero from the menu

- Change measurement settings

  - On the bottom left, click on the values displayed and change information to

    - 1 measurement per second

    - End measurement manually  (instead of after 15 sec)

## Start the measurement

We will start the computer recording before connecting the battery to the circuit. We do not want to miss the “first” measurement. There should be a 2-3 second delay between starting the data collection and connecting the battery.

- Hit “collect” (top center of display; word will change to “stop”)

- Wait 2 seconds

- Connect colored wire to “other” side of battery

- You should see that data values (non-zero) are being collected now

- Autozoom on both graphs (so data is easily visible to the eyes)

[this also ensures that the cursor is not hovering over the “stop” button]

- Watch the recorded data for the first 60 seconds to confirm that everything is working well

## Finish the measurement

We want to collect data for 2 hours = 7200 seconds after the highest value (“peak”). We will stop the measurement after at least 7500 seconds have passed. If your battery starts the measurement going down (in voltage) and then goes back up, you should measure for 8000 seconds.

- Check how much time has passed:

  - Click on the “end” of the drawn data line and it will display the current time or have the data table displayed (top right “window looking” button).

  - If the value is bigger than 7500 seconds (8000 seconds), we are ready to stop the measurement

- Hit “stop” (top center of display; word will change to “collect”)

- Save the file by going to the pull down menu at the top left corner (next to the word “untitled”)

- Choose “save as” and give the file name we will be discussing in class. (Each name needs to be unique, as all data files will be saved to the same USB.)

  - Download file to USB and email to each group member.