# Unit 2: How seismic waves bend in the ground

## Multiple choice questions

What type of ground movement is caused by ‘p-waves,’ the fastest seismic wave?

- Side-to-side

- Up-and-down

- Compressional

- Rolling

The critically refracted ray is important because:

- It bends the waves at an interface back towards the surface where we can measure them

- It provides a direct two-way travel time to calculate velocity from

- It is the first wave to arrive at the surface sensors

- It goes the deepest underground

Diagrams of the subsurface are most likely to be:

- Highly detailed, based on extensive excavation and digging

- Simple, based on basic observations we can make from the surface

- In three dimensions, and include information about groundwater movement

- Entirely based on guesses

_____ is an example of a seismically FAST materials and ______ is an example of a seismically SLOW material

- Soil; Bedrock

- Sedimentary Rock; Air

- Water; Sand

- Water; Granite

Snell’s Law predicts that waves should bend back towards the ground surface when there is a 2-layered system:

- Where the upper layer is sand and the lower layer is bedrock

- Where the upper layer is sandstone and the lower layer is soil

- Where the upper layer is saturated and the lower layer is dry

- Where the upper layer is less porous and the lower layer is more porous

The t-intercept or “T1” may be obtained from:

- The first sensor of the ground surface next to the hammer seismic source

- Back-projecting the slope of V2 on the travel-time/offset plot

- Finding the first time at which the headwave displays a refraction velocity

- The first sensor that records a reflection event

## Short Answer Questions

- Explain the steps that would be used to calculate the velocity of the lower layer using a travel-time/offset plot.

- If V1 = 500 m/s and V2 = 1000 m/s and the thickness of the top layer is 5 meters, would a signal measured at 5 meters from the seismic source be a ground wave or a refraction? At 20 m?

- Calculate a plausible incident angle when the ray is critically refracted for a subsurface where layer 1 material is soil and the layer 2 material is bedrock.

- Draw a diagram of a two-layer subsurface and assign thickness and velocities to each layer. Then, draw the following ray paths: reflected ray, critically refracted ray, ground wave, and transmitted ray.

- Explain why a refracted ray can arrive at a sensor before the direct ground wave even though the total distance that the refracted ray traveled in the subsurface is longer.