# MATLAB Audio Noise Removal Starter Script

%% MATLAB Audio Noise Removal Activity

% Student starter script

%

% Name:

% Student ID:

% Date:

%

% Purpose

% Record your own voice, add a controlled high-frequency noise signal,

% analyse the signals in the time and frequency domains, and design an FIR

% low-pass filter that reduces the noise while preserving voice clarity.

%

% Instructions

% 1. Complete every section marked TODO.

% 2. Keep an unchanged copy of the original voice recording.

% 3. Label every figure with a title, axis labels, units and a legend where

%    necessary.

% 4. Test at least three filter configurations.

% 5. Use headphones at a low and comfortable volume.

clear;

clc;

close all;

%% Part 1 Set the recording parameters

Fs = 16000;               % Sampling frequency in hertz

bitsPerSample = 16;       % Recording bit depth

numberOfChannels = 1;     % Mono recording

recordingDuration = 7;    % Recording duration in seconds

fprintf('Sampling frequency: %d Hz\n', Fs);

fprintf('Nyquist frequency: %.1f Hz\n', Fs/2);

fprintf('Recording duration: %.1f seconds\n', recordingDuration);

%% Part 2 Record your voice

% Speak clearly and maintain a consistent distance from the microphone.

% Do not include private or sensitive information in the recording.

recObj = audiorecorder(Fs, bitsPerSample, numberOfChannels);

disp('Recording will begin.');

disp('Speak now...');

recordblocking(recObj, recordingDuration);

disp('Recording complete.');

voice = getaudiodata(recObj, 'double');

numberOfSamples = length(voice);

actualDuration = numberOfSamples/Fs;

% Retain an unchanged copy for later comparison.

originalVoice = voice;

%% Part 3 Listen to and save the original recording

% Reduce the computer volume before playback.

soundsc(originalVoice, Fs);

pause(actualDuration + 1);

studentID = 'StudentID';  % TODO: Replace with your student ID.

originalFilename = [studentID '_OriginalVoice.wav'];

audiowrite(originalFilename, originalVoice, Fs);

%% Part 4 Plot the original voice in the time domain

time = (0:numberOfSamples-1)/Fs;

figure('Name', 'Original Voice');

plot(time, originalVoice);

grid on;

xlabel('Time (s)');

ylabel('Amplitude');

title('Original Voice Signal in the Time Domain');

%% Part 5 Analyse the original voice using the FFT

% TODO:

% - Calculate the FFT of the original voice signal.

% - Create the correct single-sided magnitude spectrum.

% - Create the frequency axis in hertz.

% - Plot the spectrum and identify the main useful voice-frequency range.

%

% Suggested output variable names:

% originalFFT, originalMagnitude, frequency

if false

% Write your FFT analysis code here.

figure('Name', 'Original Voice Spectrum');

% Write your plot command here.

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude');

title('Single-Sided Spectrum of the Original Voice');

end

%% Part 6 Generate and add high-frequency noise

% TODO:

% - Select a noise frequency below the Nyquist frequency but above most of

%   the useful voice-frequency range.

% - Select a noise amplitude that is clearly noticeable but does not cause

%   clipping.

% - Generate a sinusoidal noise signal with the same dimensions as voice.

% - Add the noise to originalVoice to create noisyVoice.

% - Check the maximum absolute amplitude and normalise only if required.

if false

noiseFrequency = [];  % TODO: Enter the selected frequency in hertz.

noiseAmplitude = [];  % TODO: Enter the selected amplitude.

% Write the noise-generation equation here.

noise = [];

% Write the signal-addition equation here.

noisyVoice = [];

fprintf('Noise frequency: %.1f Hz\n', noiseFrequency);

fprintf('Noise amplitude: %.3f\n', noiseAmplitude);

end

%% Part 7 Analyse and play the noisy voice

% TODO:

% - Plot the noisy signal in the time domain.

% - Calculate and plot its single-sided magnitude spectrum.

% - Confirm that a spectral peak appears at the selected noise frequency.

% - Play the noisy signal at a safe volume.

if false

figure('Name', 'Noisy Voice');

subplot(2,1,1);

% Plot noisyVoice against time.

grid on;

xlabel('Time (s)');

ylabel('Amplitude');

title('Noisy Voice Signal in the Time Domain');

subplot(2,1,2);

% Calculate and plot the noisy-voice spectrum.

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude');

title('Single-Sided Spectrum of the Noisy Voice');

% Uncomment after noisyVoice has been created.

% soundsc(noisyVoice, Fs);

% pause(actualDuration + 1);

end

%% Part 8 Design and test FIR low-pass filters

% TODO:

% Test at least three filter configurations. Change one parameter at a time

% so that its effect can be identified. Record the following for each trial:

% - filter order

% - cutoff frequency

% - window type

% - observed noise reduction

% - effect on useful voice information

%

% You may use fir1 to design the filter and freqz to examine its frequency

% response. Confirm whether your chosen fir1 syntax expects the cutoff in

% hertz or as a normalised frequency.

if false

filterOrder = [];      % TODO

cutoffFrequency = [];  % TODO

% TODO: Select or create the window vector.

selectedWindow = [];

% TODO: Design the FIR low-pass filter coefficients.

filterCoefficients = [];

% TODO: Apply the FIR filter to noisyVoice.

filteredVoice = [];

% TODO: Display the filter frequency response.

figure('Name', 'Selected FIR Filter Response');

% Add the required frequency-response command.

end

%% Part 9 Compare the original noisy and filtered signals

% TODO:

% - Compare the three time-domain waveforms using consistent axes.

% - Compare the three single-sided spectra using consistent axes.

% - Listen to the filtered voice.

% - Explain whether useful voice information was reduced.

% - Justify the filter selected as the best design.

if false

figure('Name', 'Signal Comparison');

subplot(3,1,1);

% Plot originalVoice.

grid on;

title('Original Voice');

subplot(3,1,2);

% Plot noisyVoice.

grid on;

title('Noisy Voice');

subplot(3,1,3);

% Plot filteredVoice.

grid on;

title('Filtered Voice');

xlabel('Time (s)');

% Create a second figure comparing the three frequency spectra.

% Uncomment after filteredVoice has been created.

% soundsc(filteredVoice, Fs);

% pause(actualDuration + 1);

end

%% Part 10 Save the final filtered audio

% TODO: Run this section only after filteredVoice has been created.

if false

filteredFilename = [studentID '_FilteredAudio.wav'];

audiowrite(filteredFilename, filteredVoice, Fs);

fprintf('Filtered audio saved as %s\n', filteredFilename);

end

%% Final checks

% Confirm that:

% - the script runs from beginning to end without unresolved errors;

% - all TODO sections have been completed;

% - all figures are correctly labelled;

% - the same noisy signal was used for every filter trial;

% - the final filter choice is supported by graphical and audible evidence;

% - both the original and filtered voice files have been saved.

disp('Starter sections complete. Continue by completing every TODO section.');