Landslide Mapping Protocol

Erich Herzig

Bare Earth Lidar is typically provided as a raster of elevation values. It is difficult to identify landslide features using this raster alone, and thus the raster should be processed into more informative formats. We suggest creating a slope map and using it to identify landslides. Additionally we suggest that the slope map be displayed in grayscale color with high slopes represented as black. This slope map will then highlight key landslide features such as scarps, cracks and hummocky terrain. One potential downside of using a slope map is that it lacks information about the orientations of the slope. To minimize this issue, the map can be made transparent and overlain on top of the original elevation raster (displayed in multiple-colors to accent the grayscale slope map). This coloring should help interpret the orientation of the landslide features.

Hillshade maps may be used instead of or along with slope maps. These maps are created by modelling where shadows would fall on the landscape based on a set sun angle. While these maps appear closer than slope maps to what the real landscape may look like, they can mask features that are visible in the slope map. This is because of how the sun angle inherently highlights slopes at certain aspects and can wash out slopes at other aspects. Due to this, we recommend that if hillshades are being used, multiple hillshade maps with different sun angles should be created and interpreted before a landslide inventory of a region should be considered complete.

Ideally, we would recommend using a slope elevation map to perform most mapping. Then, for uncertain or high priority landslides, use hillshade maps and aerial imagery to refine or confirm the previously mapped landslide features. Finally, any remaining uncertainties should be resolved using field based mapping of the features.

Figure 1: Headscarps (yellow) of some landslides

For identifying deeps seated landslides, the best feature to start looking for are headscarps. These will appear as linear or arced segments with high slopes near the top of a hillslope. These areas will be thin in the downslope direction, but vary in along slope length. Typically, they will also have some curvature, at least at their ends, which should help define the overall shape of the landslide. Take care to distinguish headscarps from drainage heads. These can have similarly arced steep slopes and can be almost indistinguishable from drainages that have formed in old landslides.

Looking downslope from the headscarp, there should be a landslide deposit. This will appear as an area with an overall flatter slope that the neighboring hillside with a steep toe at its downslope end and a steep headscarp at its uphill end. The deposit itself should be rough and hummocky, which should

Figure 2: Some landslides (black lines) in an urban area

appear as a varied texture in the slope map.

The exact bounds of this deposit can be difficult to map depending on the terrain. If there is a river or other body of water at the base, some or all of the deposit may be eroded away or just be underwater. In this case, we just map the lower boundary of the deposit as where it contacts the water. We follow a similar procedure for boundaries obscured by roads or other manmade structures. In older slides, the sides of the deposit tend to develop into tributaries that drain the deposit. While these tributaries erode into the sides of the deposit and can make its exact boundary unclear, a good first estimate of the deposit boundary is to use the tributary itself as the boundary. In urban environments, the deposit could have been fully or partly removed by humans. Alternately, the rough texture of the slide may be wholly or partly masked by overriding development.  We find however, that landslide deposits are often still visible in the Lidar, even if their exact boundaries tend to be obscured.

To further guide this mapping, it is useful to be aware of the typical patterns in which slide deposits appear. For many deep seated landslides, there is relatively little deformation of the slide body, and so the landslide deposit is typically takes the form of an oval or rounded rectangle (depending on how arcuate the headscarp is). These deposits will be roughly the same width as their headscarp and will have moved roughly the same distance across their whole length (moving slightly more in the center of the landslide). Some slides however may deform more depending on their style of failure or topographic setting. For example, a landslide into a concave shaped part of a valley may converge and form a narrower and longer deposit. Similarly, a landslide on a convex part may spread out to form a wider deposit with less runout. Additionally, some landslides exhibit flow like behavior in failure, these slides tend to have deposits that narrow below the headscarp and then widen again at the base of the hillslope.

Figure 3: A complex landslide

For landslide complexes, it can be difficult to distinguish separate slides. We recommend mapping complexes based what will be most beneficial to the intended use of the map. If the map will be used to calculate values like roughness based on the mapped landslide deposits, then you should try to separate as many slides as possible into different polygons, since they may have different values that may be more useful as separate data points. If instead your main goal is just to identify the location of landslides over a regional scale, it would be most useful to just map landslide complexes as a single landslide to prevent confusion due to mapping of internal secondary slides.