# Landslides and Landslide Mapping Module Part 2:

# The second part of the Landslides and Landslide Mapping module will focus on an analysis of deep-seated landslides in Seattle, WA. This builds on Part 1 as you will apply your new knowledge and techniques to consider LSI for the new region. You will also learn to map landslides on lidar yourself and will create a landslide inventory for the region. Please read through the entire exercise before beginning.

# Now that you have a mapped landslide inventory, you will analyze the data.

# As a final summative assessment of part 2, students will write an independent report that summarizes and discusses their Seattle landslide mapping and analysis.  (see details in 2 supporting documents: Seattle_Landslides_Report_Template.docx; Part_2_writtenreport_rubric.xlsx).

For this part of the assignment you will work individually (unless your instructor asks you to work in pairs or groups) to analyze the landslide inventory for Seattle. Results from this part of the module will be included and discussed in the final written report.

# Characteristics of Seattle Landslides in Inventory

Next we will perform analyses on the inventory of important characteristics such as landslide size, roughness, and failure style. We will populate several fields in the SeattleSlidePolys attribute table with this new information.

Landslide Area – First, we calculate the area of each landslide. To do this, open the attributes table of a landslide polygon by right clicking on the polygon in the table of contents and selecting `Attributes Table'. Once the attributes table has opened, click the icon on the upper left and select add field.

Name the new field ‘Area’, change the type to double and increase the precision to

15 and the scale to two. Click OK.

Right click on the new field ‘Area’ and select Calculate Geometry. Make sure the `property' is set to area. If you keep clicking ok through the boxes opened, the area of the chosen slide in square feet should be calculated and populate the area column in the attributes table.

Landslide Roughness – In order to assess roughness of each landslide (a useful relative age indicator) we will calculate the standard deviation of the slope map.

Do this by using the focal statistics tool (ArcToolbox>Spatial Analyst > Neighborhood >FocalStatistics) and calculating the standard deviation in a 10 x 10 cell (30 x 30 ft) moving window across the slope map. Remember to change the statistics type to STD.

To determine the surface roughness of each slide, access the Zonal Statistics as Table tool (Spatial Analyst>Zonal>Zonal Statistics as Table). Select the roughness map (standard deviation of slope that you just created) as your input value raster and SeattleSlidePolys as the feature zone data. Set the Zone Field to SlideName. Using the Output Table setting give your table a name. Set the Statistics type to MEAN.  Click OK.

Now you have to join the data from this table back to your mapped landslides. Do this with the join tool (Data Management>Joins>Add Join). Set the Layer Name to SeattleSlidePolys, and the Join Table to the table you just produced. Set both the Input Join Field and the Output Join Field to SlideName.

Landslide Run Out Length (H/L ratio)

# Next, we will consider landslide runout length (height of landslide over the length). To do this, we can calculate the H/L ratio, where H is defined as the elevation difference between the top of the headscarp and the base of the landslide deposit at the toe and L as the length from the headscarp to the distal edge of the landslide toe.

First, you will measure the height of the landslides. Do this with the Zonal Statistics as Table again (Spatial Analyst>Zonal>Zonal Statistics as Table). This time, select the elevation data as your input value raster and SeattleSlidePolys as the feature zone data. Set the Zone Field to SlideName. Using the Output Table setting give your table a name. Set the Statistics type to RANGE (this will give the difference between the max and min values).  Click OK.

Once again you have to join the data from this table back to your mapped landslides. Do this with the join tool (Data Management>Joins>Add Join). Set the Layer Name to SeattleSlidePolys, and the Join Table to the new table of range data you just produced. Set both the Input Join Field and the Output Join Field to SlideName.

Next, we will calculate the length of the landslides. You should do this with the measure tool. Select the measure tool on your toolbar (it looks like a ruler with two arrows above it). A window should pop-up. Select the line button in the top left of the window. Select the top of you’re the headscarp of your landslide and then double click at the distal edge of the slide toe. The measure window will then list the length of this line you just drew. Record this value for each landslide mapped and add it to a new field in your attribute table called ‘Length’ –use the same procedure as you did above when adding fields to your polygons. Save your work.

Create a new field called ‘H_L’ using the same procedure as above. Right click on the new field ‘H_L’ and select ‘Field Calculator’. A window like the one below will pop-up. Click the ‘Height’ field, the ‘/’ symbol and then the ‘Length’ field. Then click OK.

Be sure to save your work after populating the field.

Landslide Geology

It might also be useful to know the geologic unit at the surface of each landslide.

To find this information, we will compare the landslide polygons to the mapped Geology polygons for the city of Seattle.

First, add the geology layer for Seattle called ‘Geologic Units 100K’

Next, create a new field called ‘SlideGeo’, this should be a text field and allow up to 50 characters

To populate this field, do a visual examination of each landslide and the mapped geology unit at the surface. Put that unit name in the field. Choose the dominant unit if the landslide overlaps more than one.

Be sure to save your work after populating the field.

Landslide Failure Style

The last field to add and populate relates to the style of failure for each landslide. Create a new field called ‘LStyle’, this should be a text field with up to 50 characters.

Analyze each landslide and, based on the features that you see and its morphology, enter one of the four following categories: translational, rotational, flow, or complex (meaning more than one slide type is indicated for the landslide).

Be sure to save your work after populating the field.

# Merged Seattle Landslides Inventory Products

- Once you have completed the above analysis on your mapping area, you will share your mapped polygon feature class with a TA who will merge together landslide mapping from each student into a comprehensive landslide map for the entire study area that they redistribute to you as a feature class called MergedSeattleSlidePolys.

- Once you have this file, add it to your map and turn off your individual mapping. You should also add the SeattleFault feature class, which shows the bounds of the Seattle fault zone. Next, prepare a map for export as you did with your individual mapping above, but this map will be over the larger area and will include the comprehensive landslide mapping. In addition to Seattle Fault, you should also show the boundaries of the individual mapping areas. This exported map should contain all of the features added for your individual map export (such as north arrow, scale bar, etc.) and will be Figure 3 in your final written report.

- Export the MergedSeattleSlidePolys Attribute Table to an Excel file. Open the attribute table, click the icon on the upper left and select ‘Export…’. Give your output table a name and set it to be saved as a text file. Then press OK. To load the file in Excel, select Data > Get External Data > From Text. Navigate to your exported attribute table and select OK. In the window that pops up, click Next and then select ‘Comma’ in the Delimiters box. Finally, select Finish and then OK and your data should appear in the Excel sheet. This table should be included in your final report as Table 1.

# LSI Analysis - Seattle Landslides

# For this part of your analysis, you should refer to Landslide_Module_Part1.docx and repeat the LSI steps for Seattle using four factors: elevation, slope, mean annual precipitation, aspect.

# 1. You have elevation and slope maps for Seattle already. You can add a precipitation raster to your map by clicking ‘Add Data’ and then selecting ‘Sea_30yrMAP’ and then OK. You should then create an aspect map using the ‘Aspect’ tool (Spatial Analyst > Surface > Aspect) with the elevation data as your input raster.

# 2. You will also need to convert your new merged Seattle landslide inventory into a points dataset. To do this use the ‘Feature to Point’ tool (Data Management > Features > Feature To Point) with the MergedSeattleSlidePolys Feature as the ‘Input Feature’.

3. Complete the Susceptibility Analysis as you did in Part 1 and you will have two tables to add to your final report: Table 2 (Seattle LSI values for classification) & Table 3 (Seattle LSI results).