# Salmon Falls Landslide, ID: Topography Dataset

Ian Lauer, Idaho State University (laueian@isu.edu)

This is a short description of the Salmon Falls Creek data set, which is prepared for Unit 2.1 of the module “High Precision Positioning with Static and Kinematic GPS/GNSS”. The Salmon Falls landslide is the modern reactivation of a series of mass-movement features in the Sinking Canyon area of Salmon Falls Creek, Idaho. It became active in 1998 and has been moving continuously at an average rate of 10-14 cm/yr up to the present. It has two static GPS surveys in 2003 and 2017, several repeat RTK surveys of 50+ points, and multiple repeat LiDAR scans.

Location: Salmon Falls Creek, Buhl, Idaho

Appropriate for: Unit 2.1 Student Exercise: Creating Topographic Surfaces in ArcMap

OR Unit 2.2 Assignment: Change Detection with Kinematic GNSS

Data Collected:

All relevant data is contained in the SalmonFallsAssingment.mxd map document, ignore the Data folder and SallmonFallsDatabase.gdb unless data links are corrupted in the map document.

- Two LiDAR scans from 2002 and 2010

- GPS locations of monument pins in 2015 from an RTK survey

Instrument:

- LiDAR taken from an airborne platform cataloged by the Idaho LiDAR Consortium

- 2015 surveys competed using RTK GPS surveys with Leica GNSS systems

Processing:

LiDAR is provided as a cleaned, bare-earth image, which was retrieved directly from its archive. GPS points were acquired using an RTK survey with base station position corrected using OPUS.

Instructions for Use:

This data can be used by a field course that would like to use Unit 2.1 but does not have access to an appropriate field site or equipment. In addition, advanced classes with skills in ArcMap may move through this too quickly and should consider adding change detection to the assignment. If you will not teach both parts, remove data which applies to the second half to avoid confusing students (2002 DEM and 2002-2010 Difference Map).

Creating Topography: For creating topography, a class will use several different sampling designs to sample locations from the 2010 LiDAR dataset (SalFalls_2010_dem.tif) in the SalmonFallsAssignment.mxd Arcmap document. Students will then produce interpolated surface models for each samplings design and compare the precision of designs to the actual topography. GPS_Points shapefile shows the location of monuments that currently exist on the landslide and can be used as the “Informed Sampling Design” but it is encouraged that student make their own.

In addition to the 2010 DEM, a hillshade and slope map are provided in the .mxd file for visualization.

Change Detection: For advanced classes that may need additional material, you may consider utilizing the repeat LiDAR datasets to synthetically produce change detection data. To utilize the prepared data set, students can continue the previous sampling exercise by resampling the 2002 DEM with the same sampling design they made previously. They may then compare the elevations sampled from the 2002 and 2010 DEMs and then calculate vertical change in position.